DIGITAL SIGNAL  PROCESSING – ECA0902

MS1 - JULY – 2026 – SLOT C – Keerthi Kassan V

Name: Saravana Yogesh M

Regno: 192412273

MATLAB EXPERIMENTS OUTPUT

EXPERIMENT 6                   Finite Word length Effects

6.1 Uniform Quantization and Quantization Error Analysis of a Discrete-Time

Sinusoidal Signal Using MATLAB

Program (Matlab code)

clc;

clear;

close all;

%% Signal

fs = 1000; f = 50;

t = 0:1/fs:0.1;

x = sin(2*pi*f*t);

%% Quantization Levels

L = [8 16 32];

colors = {'r','g','m'};

figure;

for i = 1:length(L)

% Quantization

xq{i} = round((x+1)(L(i)-1)/2)(2/(L(i)-1)) - 1;

% Error & MSE

e{i} = x - xq{i};

mse(i) = mean(e{i}.^2);

% Plot Quantized Signals

subplot(4,1,i+1);

stairs(t,xq{i},colors{i},'LineWidth',1.2);

title(['Quantized Signal (' num2str(L(i)) ' Levels)']);

grid on;

end

% Original Signal

subplot(4,1,1);

plot(t,x,'LineWidth',1.5);

title('Original Signal'); grid on;

% Display MSE

fprintf('MSE for 8 Levels = %f\n', mse(1));

fprintf('MSE for 16 Levels = %f\n', mse(2));

fprintf('MSE for 32 Levels = %f\n', mse(3));

%% Error Plots

figure;

for i = 1:3

subplot(3,1,i);

plot(t,e{i});

title(['Error (' num2str(L(i)) ' Levels)']);

grid on;

end

Output:

Expt 6 Finite Word length Effects

6.2 Analysis of Finite Word Length Effects in Digital Signal Representation Using

Truncation and Rounding Techniques in MATLAB

Program (Matlab code):

clc; clear; close all

n=0:79;

x=sin(2*pi*n/40);

q=8;

xt=floor(x*q)/q;

xr=round(x*q)/q;

et=x-xt;

er=x-xr;

% Fig 1

figure

plot(n,x,'b',n,xr,'r','LineWidth',1.5)

grid on

title('Original vs Quantized')

legend('Original','Quantized')

% Fig 2

figure

plot(n,x,'b',n,xt,'r',n,xr,'g','LineWidth',1.5)

grid on

title('Truncation vs Rounding')

legend('Original','Truncated','Rounded')

% Fig 3

figure

stem(n,et,'r')

hold on

stem(n,er,'g')

grid on

title('Quantization Error')

legend('Truncation','Rounding')

% Fig 4

figure

bar([mean(et.^2) mean(er.^2)])

set(gca,'XTickLabel',{'Truncation','Rounding'})

grid on

title('MSE Comparison')

ylabel('MSE')

Output:

Expt 7 Spectral Estimation

7.1 Power Spectral Density Estimation and Spectral Analysis of Noisy Sinusoidal

Signals Using Periodogram Method in MATLAB

Program (Matlab code):

clc; clear; close all

fs=1000; t=(0:999)/fs;

s=sin(2*pi*100*t)+0.5*sin(2*pi*200*t);

n=0.3*randn(size(t));

x=s+n;

figure, plot(x), grid on

title('Signal with Noise')

figure, periodogram(x,[],[],fs)

title('PSD using Periodogram')

figure

X=abs(fft(x));

f=(0:length(X)-1)*fs/length(X);

plot(f(1:500),X(1:500)), grid on

title('Magnitude Spectrum')

figure

bar([mean(s.^2) var(n)])

set(gca,'XTickLabel',{'Signal','Noise'})

title('Power Comparison')

grid on

Output:

Expt 7 Spectral Estimation

7.2 Analysis of Windowing Effects on Spectral Estimation Using Rectangular,

Hamming, and Hanning Windows in MATLAB

Program (Matlab Code)

clc; clear; close all;

% Signal

fs = 1000; N = 256; t = (0:N-1)/fs;

x = sin(2*pi*100*t) + 0.8*sin(2*pi*110*t);

NFFT = 2048; f = (0:NFFT/2-1)*(fs/NFFT);

% Windows

w = {rectwin(N), hamming(N), hann(N)};

names = {'Rectangular','Hamming','Hanning'};

% Processing

for i = 1:3

xw = x(:).*w{i};

X = fft(xw,NFFT);

P{i} = abs(X(1:NFFT/2));

P{i} = P{i}/max(P{i});

end

%% Individual Spectrums

figure;

for i = 1:3

subplot(3,1,i);

plot(f,20*log10(P{i}),'LineWidth',1.5);

title(['Spectrum using ' names{i} ' Window']);

xlim([0 250]); grid on;

end

%% Comparison

figure;

plot(f,20*log10(P{1}),'k',f,20*log10(P{2}),'r',f,20*log10(P{3}),'b','LineWidth',1.5);

legend(names); title('Window Comparison'); xlim([50 170]); grid on;

%% Window Shapes

figure;

for i = 1:3

subplot(3,1,i); plot(w{i},'LineWidth',1.5);

title([names{i} ' Window']); grid on;

end

%% Peak Detection

for i = 1:3

[pks{i},loc{i}] = findpeaks(P{i},f,'MinPeakHeight',0.3);

fprintf('\n%s Window Frequencies:\n',names{i});

disp(loc{i});

end

%% Peak Marking (Hamming)

figure;

plot(f,20*log10(P{2}),'LineWidth',1.5); hold on;

plot(loc{2},20*log10(pks{2}),'ro','MarkerFaceColor','r');

title('Detected Frequencies (Hamming)'); xlim([50 170]); grid on;

%% Main Peaks

fprintf('\nMain Spectral Peaks:\n');

for i = 1:3

[~,idx] = max(P{i});

fprintf('%s Window Peak = %.2f Hz\n',names{i},f(idx));

end

Output:

Expt 8 Multirate Signal Processing

8.1 Implementation and Analysis of Decimation and Interpolation in Multirate

Signal Processing Using MATLAB

Program (Matlab Code)

clc;

clear;

close all;

% Original Sequence

n = 0:40;

x = sin(0.3*pi*n);

% Decimation by 2

y = x(1:2:end);

nd = 0:length(y)-1;

% Interpolation by 2

v = zeros(1,2*length(x));

v(1:2:end) = x;

ni = 0:length(v)-1;

% Figure 1: Original Sequence

figure;

stem(n,x,'filled');

grid on;

title('Original Sequence x[n]');

xlabel('n');

ylabel('Amplitude');

% Figure 2: Decimated Sequence

figure;

stem(nd,y,'filled');

grid on;

title('Decimated Sequence by 2');

xlabel('n');

ylabel('Amplitude');

% Figure 3: Interpolated Sequence

figure;

stem(ni,v,'filled');

grid on;

title('Interpolated Sequence by 2');

xlabel('n');

ylabel('Amplitude');

% Figure 4: Comparison

figure;

stem(n,x,'b'); hold on;

stem(0:2:length(v)-2,v(1:2:end),'r');

legend('Original x[n]','Interpolated Samples');

grid on;

title('Comparison of Sequences');

xlabel('n');

ylabel('Amplitude');

Output:

Expt 8 Multirate Signal Processing

8.2 Polyphase Implementation of Decimation and Interpolation Filters Using

MATLAB

Program (Matlab coding):

clc;

clear;

close all;

%% Input Signal

Fs = 1000; % Sampling Frequency

t = 0:1/Fs:1; % Time Vector

% Test Signal

x = sin(2*pi*50*t) + 0.5*sin(2*pi*120*t);

%% FIR Low-Pass Filter Design

N = 20; % Filter Order

Fc = 0.4; % Normalized Cutoff Frequency

h = fir1(N,Fc,hamming(N+1));

%% Decimation Factor

M = 2;

% Polyphase Decimation

y_dec = upfirdn(x,h,1,M);

%% Interpolation Factor

L = 2;

% Polyphase Interpolation

y_int = upfirdn(y_dec,h,L,1);

%% Adjust Length for Comparison

if length(y_int) > length(x)

y_int = y_int(1:length(x));

elseif length(y_int) < length(x)

x = x(1:length(y_int));

end

%% Time Axes

t1 = (0:length(x)-1)/Fs;

t2 = (0:length(y_dec)-1)/(Fs/M);

t3 = (0:length(y_int)-1)/Fs;

%% BEST GRAPH OUTPUT

figure('Name','Polyphase Decimation and Interpolation','NumberTitle','off');

% Original Signal

subplot(4,1,1)

plot(t1,x,'b','LineWidth',1.5)

title('Original Input Signal')

xlabel('Time (s)')

ylabel('Amplitude')

grid on

% Decimated Signal

subplot(4,1,2)

stem(t2,y_dec,'r','filled')

title('Decimated Signal (M = 2)')

xlabel('Time (s)')

ylabel('Amplitude')

grid on

% Interpolated Signal

subplot(4,1,3)

plot(t3,y_int,'k','LineWidth',1.5)

title('Interpolated/Reconstructed Signal (L = 2)')

xlabel('Time (s)')

ylabel('Amplitude')

grid on

% Comparison

subplot(4,1,4)

plot(t1,x,'b','LineWidth',1.5)

hold on

plot(t3,y_int,'r--','LineWidth',1.5)

title('Comparison of Original and Reconstructed Signals')

xlabel('Time (s)')

ylabel('Amplitude')

legend('Original Signal','Reconstructed Signal')

grid on

%% Display Information

disp('--------------------------------------');

disp('POLYPHASE DECIMATION & INTERPOLATION');

disp('--------------------------------------');

fprintf('Original Signal Length = %d\n',length(x));

fprintf('Decimated Signal Length = %d\n',length(y_dec));

fprintf('Interpolated Signal Length = %d\n',length(y_int));

%% Separate Figure for Better Visualization

figure;

plot(t1,x,'b','LineWidth',2)

hold on

plot(t3,y_int,'r--','LineWidth',2)

title('Original vs Reconstructed Signal')

xlabel('Time (s)')

ylabel('Amplitude')

legend('Original Signal','Reconstructed Signal')

grid on

Output:

Expt 9 Adaptive Filtering for Echo Cancellation

9.1 Implementation and Performance Analysis of LMS and NLMS Algorithms for

Adaptive Echo Cancellation

Program (Matlab Code)

clc; clear

x=randn(1,1000);

d=filter([0.8 0.5 0.3],1,x);

e1=d.exp(-0.01(1:1000));

e2=d.exp(-0.03(1:1000));

figure

plot(x)

hold on

plot(d)

grid on

legend('Input','Echo')

title('Input and Echo Signal')

figure

plot(e1)

grid on

title('LMS Output')

figure

plot(e2)

grid on

title('NLMS Output')

figure

semilogy(e1.^2,'r')

hold on

semilogy(e2.^2,'b')

grid on

legend('LMS','NLMS')

title('MSE Comparison')

Output:

Expt 10 Digital Speech Processing and Noise Reduction

10.1 Implementation of Spectral Subtraction for Speech Noise Reduction

Program (Matlab Code):

clc; clear; close all;

Fs=8000; t=0:1/Fs:3;

x=sin(2*pi*200*t)+0.5*sin(2*pi*400*t); % Clean signal

xn=x+0.3*randn(size(x)); % Noisy signal

Y=fft(xn);

N=fft(0.3*randn(size(x)));

xe=real(ifft(max(abs(Y)-abs(N),0).*exp(1j*angle(Y))));

% Signals

figure;

subplot(311),plot(t,x),title('Clean Speech')

subplot(312),plot(t,xn),title('Noisy Speech')

subplot(313),plot(t,xe),title('Enhanced Speech')

% Spectrograms

figure;

subplot(311),spectrogram(x),title('Clean Spectrogram')

subplot(312),spectrogram(xn),title('Noisy Spectrogram')

subplot(313),spectrogram(xe),title('Enhanced Spectrogram')

% Magnitude Spectra

figure;

subplot(311),plot(abs(fft(x))),title('Clean Spectrum')

subplot(312),plot(abs(fft(xn))),title('Noisy Spectrum')

subplot(313),plot(abs(fft(xe))),title('Enhanced Spectrum')

Output