SIMATS ENGINEERING

CO3- ASSESSMENT TOOL 2

SIMULATION TASK

COURSE NAME: DIGITAL SIGNAL PROCESSING	COURSE CODE: ECA09

NAME: Saravana Yogesh M

REGNO: 192412273

# COURSE OUTCOME COVERED

CO 3: Evaluate finite word-length effects and multirate signal processing techniques, including decimation, interpolation, and polyphase decomposition. Bloom Level mapped- BL3,BL4,BL5

Topics Covered:

Quantization- Truncation and Rounding errors - Quantization noise - Limit cycle oscillations

– Signal scaling.Parametric and Non parametric  Spectral estimation. DecimationInterpolation-Sampling rate conversion-Implementation of sampling rate conversionPolyohase Decomposition -5G Decimation/Interpolation: For mmWave beamforming.

| Assessment Tool number | : CO3-AT2 |
| --- | --- |
| Assessment Tool Weightage | : 35% |
| Assessment Tool Name | : Simulation Task |
| Duration of this assessment | : 60 Minutes |
| Marks | : 25 Marks |
| Type of Assessment Conduction | : Self Learning (Home Task) |

# Beamforming Signal Rate Conversion

Simulate sampling rate conversion of a beamformed signal and verify preservation of phase alignment. (25) (BL5) (WK4) Given Data:

- Input Sampling Rate = 61.44 MHz

- Output Sampling Rate = 30.72 MHz Tasks:

- Implement rate conversion.

- Analyze phase response.

- Measure amplitude distortion.

- Compare spectra.

- Verify beamforming integrity.

- Discuss performance.

Code of Conduct:

I  Saravana Yogesh M, 192412273  (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

RUBRICS FOR CALCULATION:

| Simulation tools |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightag e (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Conceptual Understanding | 20% | Demonstrates strong understanding of underlying concepts in the simulation | Shows good understanding with minor gaps | Partial understanding; some misconceptions | Lacks understanding of key concepts |
| Model Design | 25% | Develops accurate, wellstructured simulation/mode l independently | Develops correct model with minor errors | Model is incomplete or requires guidance | Unable to develop a proper model |
| Tool Usage | 20% | Uses simulation tools effectively with correct setup and execution | Uses tools with minor errors or guidance | Limited ability to use tools properly | Unable to use simulation tools |
| Analysis of Results | 20% | Provides clear, logical analysis and meaningful interpretation of results | Analysis is reasonable with minor gaps | Superficial analysis; limited interpretation | Unable to analyze or interpret results |
| Documentatio n | 15% | Presents results clearly with proper documentation and structured reporting | Documentatio n is adequate with minor issues | Poorly organized or incomplete documentation | No proper documentation or presentation |

SOLUTION:

Matlab Code:

clc;

clear;

close all;

%% =========================================================

%  SAMPLING RATE CONVERSION OF A BEAMFORMED SIGNAL

%  Input Fs  = 61.44 MHz

%  Output Fs = 30.72 MHz

%  Conversion = 2:1 Decimation

%% =========================================================

%% 1. System Parameters

Fs_in  = 61.44e6;       % Input sampling frequency

Fs_out = 30.72e6;       % Output sampling frequency

R = Fs_in/Fs_out;

fprintf('Input Sampling Rate  = %.2f MHz\n', Fs_in/1e6);

fprintf('Output Sampling Rate = %.2f MHz\n', Fs_out/1e6);

fprintf('Sampling Rate Ratio  = %.0f:1\n', R);

%% 2. Simulation Parameters

T = 20e-6;                         % Simulation duration

t = (0:1/Fs_in:T-1/Fs_in).';       % Input time vector

%% 3. Beamformed Signal Generation

% Two signal components

f1 = 8e6;        % 8 MHz

f2 = 12e6;       % 12 MHz

% Relative phases

phi1 = 20*pi/180;

phi2 = 70*pi/180;

% Four antenna channels

% Each antenna has a different phase delay

phase_shift = [0 30 60 90]*pi/180;

x = zeros(length(t),4);

for k = 1:4

x(:,k) = cos(2*pi*f1*t + phi1 + phase_shift(k)) ...

+ 0.6*cos(2*pi*f2*t + phi2 + phase_shift(k));

end

%% 4. Beamforming

% Equal beamforming weights

w = ones(4,1)/4;

beamformed_input = x*w;

%% 5. Anti-Aliasing FIR Filter

% New Nyquist frequency = 15.36 MHz

% Choose cutoff slightly below this value

Fc = 14e6;

N = 100;

b = fir1(N, Fc/(Fs_in/2));

%% Filter the beamformed signal

beamformed_filtered = filter(b,1,beamformed_input);

%% 6. Decimation by 2

beamformed_output = beamformed_filtered(1:2:end);

t_out = (0:length(beamformed_output)-1).'/Fs_out;

%% 7. Normalize Amplitude

beamformed_output = beamformed_output / max(abs(beamformed_output));

beamformed_input_norm = beamformed_input / ...

max(abs(beamformed_input));

%% =========================================================

% 8. Time Domain Comparison

%% =========================================================

figure;

subplot(2,1,1);

plot(t*1e6,beamformed_input_norm);

xlabel('Time (\mus)');

ylabel('Amplitude');

title('Input Beamformed Signal - 61.44 MHz');

grid on;

xlim([0 2]);

subplot(2,1,2);

plot(t_out*1e6,beamformed_output);

xlabel('Time (\mus)');

ylabel('Amplitude');

title('Output Beamformed Signal - 30.72 MHz');

grid on;

xlim([0 2]);

%% =========================================================

% 9. Phase Response Analysis

%% =========================================================

% Hilbert transform gives instantaneous phase

analytic_in = hilbert(beamformed_input_norm);

phase_in = unwrap(angle(analytic_in));

analytic_out = hilbert(beamformed_output);

phase_out = unwrap(angle(analytic_out));

% Remove linear phase trend for comparison

p_in = polyfit(t,phase_in,1);

p_out = polyfit(t_out,phase_out,1);

phase_in_detrended = phase_in - polyval(p_in,t);

phase_out_detrended = phase_out - polyval(p_out,t_out);

figure;

plot(t*1e6,phase_in_detrended*180/pi,'LineWidth',1);

hold on;

plot(t_out*1e6,phase_out_detrended*180/pi,'--','LineWidth',1);

xlabel('Time (\mus)');

ylabel('Phase (degrees)');

title('Phase Alignment Before and After Sampling Rate Conversion');

legend('Input','Output');

grid on;

xlim([0 2]);

%% =========================================================

% 10. Amplitude Distortion

%% =========================================================

input_rms = rms(beamformed_input);

output_rms = rms(beamformed_output);

amplitude_distortion = ...

abs(output_rms-input_rms)/input_rms*100;

fprintf('\nAmplitude Distortion = %.4f %%\n', ...

amplitude_distortion);

%% =========================================================

% 11. Spectrum Comparison

%% =========================================================

NFFT = 2^nextpow2(length(beamformed_input));

X = fft(beamformed_input,NFFT);

Y = fft(beamformed_output,NFFT);

f_in = (0:NFFT/2-1)*Fs_in/NFFT;

f_out = (0:NFFT/2-1)*Fs_out/NFFT;

X_mag = abs(X(1:NFFT/2));

Y_mag = abs(Y(1:NFFT/2));

X_mag = X_mag/max(X_mag);

Y_mag = Y_mag/max(Y_mag);

figure;

plot(f_in/1e6,20*log10(X_mag+eps),'LineWidth',1);

hold on;

plot(f_out/1e6,20*log10(Y_mag+eps),'--','LineWidth',1);

xlabel('Frequency (MHz)');

ylabel('Magnitude (dB)');

title('Spectrum Comparison');

legend('Input Spectrum','Output Spectrum');

grid on;

xlim([0 30]);

%% =========================================================

% 12. Beamforming Integrity

%% =========================================================

% Calculate relative phase between antenna channels

phase_channel_input = zeros(4,1);

phase_channel_output = zeros(4,1);

for k = 1:4

% Input

analytic_channel = hilbert(x(:,k));

phase_channel_input(k) = ...

mean(unwrap(angle(analytic_channel)));

end

% Decimate individual antenna channels

x_output = zeros(length(t_out),4);

for k = 1:4

temp = filter(b,1,x(:,k));

x_output(:,k) = temp(1:2:end);

analytic_channel = hilbert(x_output(:,k));

phase_channel_output(k) = ...

mean(unwrap(angle(analytic_channel)));

end

% Relative phase

relative_input = phase_channel_input-phase_channel_input(1);

relative_output = phase_channel_output-phase_channel_output(1);

phase_error = relative_output-relative_input;

fprintf('\nBeamforming Phase Analysis\n');

fprintf('---------------------------------\n');

for k = 1:4

fprintf('Channel %d: Input = %.3f deg, Output = %.3f deg, Error = %.3f deg\n',...

k,...

relative_input(k)*180/pi,...

relative_output(k)*180/pi,...

phase_error(k)*180/pi);

end

%% =========================================================

% 13. Beamforming Integrity Plot

%% =========================================================

figure;

bar(1:4,[relative_input relative_output]*180/pi);

xlabel('Antenna Channel');

ylabel('Relative Phase (degrees)');

title('Beamforming Phase Alignment');

legend('Before Conversion','After Conversion');

grid on;

%% =========================================================

% 14. Performance Summary

%% =========================================================

fprintf('\n============================================\n');

fprintf('        SAMPLING RATE CONVERSION RESULT\n');

fprintf('============================================\n');

fprintf('Input Rate       : %.2f MHz\n',Fs_in/1e6);

fprintf('Output Rate      : %.2f MHz\n',Fs_out/1e6);

fprintf('Conversion       : 2:1 Decimation\n');

fprintf('Filter Cutoff    : %.2f MHz\n',Fc/1e6);

fprintf('Amplitude Error  : %.4f %%\n',amplitude_distortion);

fprintf('Maximum Phase Error = %.4f degrees\n',...

max(abs(phase_error))*180/pi);

fprintf('============================================\n');

OUTPUT:

Figure 1:

Figure 2:

Figure 3:

Figure 4: